# Purchase

## Antifreeze: 14 gal -60 & 4 gal -100

## Oil: Rotella-T SAE 15W-40 [8 qts]

## Filter: NAPA 1515 or Wix 51515

# Exterior

## Pump-out

## Remove sails

## Remove covers

## Remove screens

## Stow/clean dingy

## Remove dingy engine (service below)

## Remove davits (7/16” wrench & Phillips)

## Remove motor lift

## Remove ring buoy

## Remove EPIRB

## Remove exterior compass

## Remove seats & dry cushions (7/16” socket)

## Lower GPS antenna (5/16” hex)

## Loosen shrouds/stays (looking down – turn clockwise) – ½” wrench & large screwdriver

### Remove screws (9/32 socket & Phillips screwdriver)

### Front shrouds: 4 half turns

### Middle shrouds: 6 half turns

### Back shrouds: 2 half turns

### Rear stay: 3 full turns

## Secure lines

# Dingy engine (yard? ___)

## Fog

## Remove gas

## Oil crankcase

## Change sparkplug

# Engine (in water)

## Log engine hours

## Run engine to operating temperature

## Drain oil

## Change filter (turn counter-clockwise)

## Add oil (start @ 7 qts & use all 8)

## Check antifreeze to ____ [-34]

## Change fuel filters

### Primary: _______ (yearly)

### Secondary: ________ (10/23/05)

## Wipe down engine bilge

## Check alternator belt

## Check raw water impellor

## Grease pillow block

## Fill water in batteries (distilled)

# Fuel

## Top off tanks (____ gal each)

## Biocide (1 oz/30 gal)

# Westabo heater

## Change fuel filter

## Check antifreeze to -34°

# Air Conditioning (1.5 gal antifreeze)

## Close thru-hull

## Rinse condenser (cockpit)

## Clean & stow filter (with washer)

## Install filter-hose attachment

## Blowout pump (bilge)

## Open AC shut-off

## Blowout condenser (cockpit)

## Replace filter cover

## Antifreeze pump [1 gal]

## Close AC shut-off

## Antifreeze condenser (use funnel) [0.5 gal]

## Remove filter cover

## Spray down rear bilge

# Washdown pump (0.5 gal antifreeze)

## Remove hose

## Open spigot

## Drain line (5/16” socket)

## Antifreeze pump [0.5 gal]

## Clean & stow filter (no washer)

## Spray down front bilge

# Waste/head (3 gal antifreeze)

## Flush head lines

## Flush tank twice

## Flush macerator

## Drain tank

## Antifreeze intake lines & conditioner [5/16” socket for clamp) [0.5 gal]

## Antifreeze tank – from pump-out [1 gal]

## Antifreeze head – pump through lines [1.5 gal]

## Antifreeze macerator (run pump 10 sec)

# Shower pump (before dingy stowage)

## Run shower pump

## Antifreeze line & pump [0.5 gal]

## Clean & stow filter (no washer)

# Water (5.0 gal antifreeze)

## Shut off & drain water purifier

## Drain tanks

## Drain water heater

## Bypass water heater

## Blowout lines

## Empty galley faucet/spray and head faucet/shower valve

## Drain water purifier & reinstall

## Antifreeze hand/foot pumps [0.75 gal]

## Antifreeze lines [2 gal

## Antifreeze galley faucet/spray

## Antifreeze head faucet/shower valve

## Clean & stow pump filter

## Antifreeze tanks [2 gal each]

# Frig

## Shut down

## Clean

# Interior

## Remove binoculars

## Remove first aid kits

## Remove batteries

## Remove “work” freezables

## Remove “food” freezables

## Remove rugs & blankets

## Remove computer, if needed

## Pack up files as needed

# Propane

## Shut-off tanks

## Disconnect

# Out of water (3 gal antifreeze)

## Charge batteries

## Open seacocks

## Bilge

### Drain (remove/reinsert garboard plug – plug handle in key drawer)

### Pump dry with hand pump
(pump handle in cockpit locker)

### Antifreeze electric pump [2 gal]

### Antifreeze hand pump (cockpit) [1 gal] - pump dry

## Galley seawater pump

### Pump dry

### Antifreeze from outside [0.25 gal]
(use 7’-0” ¼” ID black tubing)

## Remove garboard plug

## Engine raw water [2¾ gal of -100]

### Run out water first

### Fill strainer & cap [0.25 gal]

## Power

### Turn off panel breakers

### Turn off engine compartment breakers

### Turn off Battery Parallel Switch

### Turn off Eng Bat Charge Switch

### Turn off Master

### Disconnect bank of 4 house batteries– check & disconnect
(use 9/16” wrench)

### Disconnect bank of 2 starting batteries – check & disconnect
(use small adjustable wrench)

## Exterior

### Close blower & heater exhaust vents

### Stow fenders

### Stow dock lines

### Secure halyards

### Touch-up varnish

## Open cockpit scuppers

## Lock cockpit lockers

## Secure whisker pole

## Shrink wrap (yard)

# Spring

## Fill propane tanks

# Questions

## Cutlass bearing inlet?

## Engine exhaust wet vent?