OPERATIONS

AND

MAINTENANCE

MANUAL

TAYANA VANCOUVER 42

TABLE OF CONTENTS

I 	INTRODUCTION 	6

II	SPECIFICATIONS 	7

General Design Specifications	7

Serial Numbers	7

Materials 	8

Construction	9

Procedures and Data for Documentation	12

III	COMMISSIONING AND DECOMMISSIONING	14

Initial Commissioning	14

Receiving and Commissioning Check List	16

Owner Commissioning		23

Decommissioning for Storage		25

Recommissioning After Layup	27

IV	OPERATING PROCEDURES	29

Sailing the Tayana Vancouver 42 Cutter	29

Tuning the Spars and Rigging	30

Tuning Under Sail	30

Handling Under Power	31

Fuel System	33

Electrical System	37

Plumbing Systems	43

Propane System	48

V	MAINTENANCE AND MAINTENANCE PROCEDURES	51

Hull	51

Deck and Cabin Top	52

Interior Woodwork	54

Spars, Rigging and Sails	54

Engine and Related Systems	58

Electrical Systems		65

Plumbing and Sanitation Systems	69

LIST OF TABLES AND FIGURES

V-42 Midcockpit Sailplan	2

V-42 Aft Cockpit Typical Interior Layout	3

Figure II - 1	Fiberglass Layup Schedule	10

Figure II - 2	V-42 Deck and Hull Joint	11

Figure IV - 1	Fuel Supply Plumbing	34

Figure IV - 2	Fluid Plumbing System	35

Figure IV - 3	Center Cockpit Tankage Diagram	36

Figure IV - 4	Electric Power Supply	38

Table IV- 1	Direct Current Energy Audit	41

Figure IV - 5	Fresh Water System	44

Figure IV - 6	Waste Water Plumbing System	45

Figure IV - 7	Partial Plumbing System	46

Figure IV - 8	Typical Propane System	49

Figure V - 1	Electrical Bonding System	68

CHAPTER I:  INTRODUCTION

Congratulations on your selection of a Tayana Vancouver 42 cruising yacht. There is no better yacht in her size on the market. With care and proper maintenance she will not only prove to be an excellent investment, but she will take you cruising anywhere in the world safely and comfortably. This is being proven daily as owners take their V-42s the world over. Ocean crossings by V-42s are routine. Circumnavigations have been reported. By the same token, over a hundred of these yachts are the primary homes of their owners.

The object of this Owners Manual/Operations and Maintenance Manual is to guide you in providing proper care and maintenance to your yacht, and thereby, help you know your yacht better. Really, it isn=t hard, and it can be fun! Your V-42 has been designed and built with maintenance in mind. Machinery, pumps, glands, seacocks, etc., have been installed to ensure full and easy access. Where simplicity and adequate function do not conflict, the simpler equipment and its placement has been chosen. In a few cases, custom interiors have complicated some equipment installations, but you will find that adequate access and straight-forward installations have alleviated the potential for problems.

In the various sections which follow, you will find discussions of materials and construction, hull maintenance, rigging and sails, machinery, electrical systems, and plumbing. Remember however, that it is a Tayana Vancouver 42 manual, it is not a general maintenance manual. It is not designed to replace the manufacturers equipment manuals provided with your equipment. In fact, should you find that there is a conflict between this manual and your manufacturer=s instruction book, the latter should take precedence.

This manual should serve as a centerpiece for your operation and maintenance documentation. It might be more serviceable when you place it in a three ring binder together with other manuals and operational data such as pertinent radio frequencies, contact information for various maintenance facilities, and other data on your specific boat.

For those who have purchased an older boat and find some of the information lacking, there are many avenues to replace this information. Your boat may have many new installations not developed when the boat was built. Again, for these equipments, you must refer to the manuals provided with the equipment.

Also, remember as you read/use this book that it was developed using a Astandard@ V-42 as its model. Therefore, there may be items which are different on your boat from that referenced herein. It would be wise for you to indicate the differences directly in the manual when you receive it. If you have had to develop drawings (electrical or plumbing for example) which you find helpful, please pass them on to TOG for distribution to other owners.

Finally, this manual was developed by TOG members who own V-42s, who maintain them and sail them. It was developed for many owners who felt that they needed a little help in working with their new boat. The final caveat from the Owners Group is that we are not responsible for the content of this manual or for any results that may arise from following the information provided in this manual.