Home power notes by

John Hovan,

Austin, TX

- Batteries (Lead Acid Batteries)

  - 2-12kwh per day go to a 24volt system

  - 700-3-4k batteries 12, 24, 48v

  - Sizing the Battery

    - Need enough power for 4 days of storage.

    - Wattage / voltage of systems x 4 days.

  - Local Action

    - Battery will discharge 4% every week on its own

  - recognizing a deep cycle battery.  Heavy. More weight is better.

    - Exception is phone system battery. Not deep cycle but heavy

    - Antimony alloyed on the plates

  - Cycle as shallow as possible

  - L16 Mfg’s

    - Interstate

    - Trojan

    - Surrette.

    - Surrette battery – 10 year warranty. Very high quality.

  - Golf Cart – Between deep cycle and fork lift.

  - Eddison Battery  is Nickel Iron battery

    - Very good battery but not available in quality battery.

  - Don’t add batteries after 2 years to a battery bank.

  - Sealed cells for sanitation and portability

    - Delicate batteries. Don’t like overcharging.

    - Sealed battery recombines hydrogen and oxygen.

  - Never contaminate cells.  Using hydrometer is bad since opening the cell all the time.

  - New battery 86-87% efficient.  60% when near end of life.

  - zero degrees batteries may have 50% of the rating.

  - Battery sulphates over time.

  - At least twice a week, batteries need to be fully charged.

  - Series and parallels wired batteries should be wired with plus on one end and minus and the other end.

  - Charge known by Ampere hours – Battery ampere hour meter

    - E-Meter by cruising equipment, volts, net, ampere hrs, #hrs,

    - Trimetric meter by Bogart engineering both are great.

    - Trace hired Trimetric guy to build TM48

    - Capacity ampere hours divided by 10 = c10 for charge rate

  - Battery should store 4 days of energy. Depth of discharge would be 10-12% daily.

  - EDTA $12 a pound.  Does not work with car batteries. De sulphating.

    - Trail Head Supply in Utah sells it.

  - Distilled water for battery.   Rain water, tap, well water etc. not ok.

    - Trojan battery One pint of water every two months.

    - Temperature plays a part.

    - Batteries working properly use water. Water evaporates in production of electricity.  Too much indicates overcharge.

  - Hydro Cap (good device, reduces hydrogen escape)

    - Looks like a mushroom.  Platinum cloth in cap.  Recycles gas.

    - If you can touch for 5-10 secs ok.  If hot, overcharged batteries.

    - Excellent indicator of gas escape.

    - Keeps acid off the top of the battery.

    - 1 pint of water every 12 months.

  - Cleaning

    - Never use baking soda to clean the battery

    - Paper towel is best.  Acid eats cloth.

  - Equalizing

    - Overcharging a fully charged battery.

    - Use stock caps and not the Hydro caps.

    - Capacity of battery in ampere hours ex 1000 divide by 20 to get maximum charge rate. 5-7 hours.  Add water before.

    - Overcharging the battery so all cells come up to a fully charged condition.

    - Every 5-7 deep cycles or every 2 months whichever comes first.

    - Pulse tec made in texas.  Product is powerpulse.

  - Safety

    - Store baking soda for battery disasters.

    - 1 pound to neutralize 1 quart of battery acid.

    - Long sleeve gloves – chemical resistant

    - Safety glasses.

    - Smoke detector

    - ADC Type fire extinguisher.

    - No smoking sign.

    - Lock on the door.

    - At least a 2 inch pipe ventilator

- Home Power Systems

  - 48 Modules 2-2.2 KW in full sun.

  - linear current booster converts 48v to 12v. (Buck Converters)

- Solar Cooking

  - Cook between 12 and 2 pm., Windy days take longer

  - Twice as long to cook in a solar oven as a regular one.

  - Water pasturization 150 degrees, 160 degrees for food.

  - Parabolic Cooker – Very high heat.  Very careful so the sun doesn’t start a fire.  Have to pay close attention to it.  Quart of water into a pyrex , put into a cooker, waited a few minutes and water was boiling.  Jiffy pop caught on fire.

  - More water content the closer the temp will stay to 212.  Takes longer.

  - Broccoli and Cauliflower doesn’t work well.  Gets mushy.

  - Good Foods

    - Corn on the cob.

    - Potatoes

    - Pies put crust in first, then put stuff into.  Same with pizza.

    - Do not use cheese on top.  Put it on after.  Turns to rubber in oven.

    - Very rarely add water.  Enough in foods.

    - Lasagna is only pasta that works.  Put noodles in dry and sauce soupy.  No lid on the pot.  Some condensation.

    - Batter breads are good.  Put rock in oven first.

  - Good to have lids on pots.  Do not cover with saran wrap.

  - Cooking with the sun (excellent book - deserts)

  - One rule – cook until it’s done.

  - Sam Erwin Solar Cooker Oregan, greg lynch contact.

  - Connot bake with parabolic cookers. It cooks.

  - Changes starches to simple sugars in solar oven.

  - Miracle maid – excellent pot.

  - Banana bread.

- Water Pumps

  - Ram Pumps

    - Folk ram pump.  Need stream or creak.

    - Snifter valve – replace air in bell

  - High lifter

    - Needs clean water source.

  - Sling Pump

    - Can be tethered mid stream.

- Wiring

  - Use vasoline on outside connections.

  - Do not bend wires 90 degrees or kink the wire.

  - Solid state relay for float switch and pump. Normally closed.

  - Charge controller disconnect and fuse on both sides.

  - Square D Fuse Controller, DC rated for voltage, Q0 breaker.

  - Charging is usually over the rated battery voltage.

  - Breaker should be 1.25% of max load.

  - 4k watt inverter, 24v, 200amp breaker plus 25% is 250 amp breaker.

  - NEC - Group all disconnects together. 36” free in front of it.  One hand one motion to shutoff the power.

  - Heinemans, airpacs 65 amps.

  - If you put PV’s on the roof of a house, NEC requires a GFI.   Trace has the only product.  Don’t put them on the roof.

  - Keep plus and minus wires together to cancel inductive effects of wiring.

  - PV interconnect is ok to not run conduit if it is UV rated.

  - Authorized personnel only – do not have to use conduit according to the rules.  As long as doors are locked.  AHJ – Authority Having Jurisdiction.

  - Non conductive fitting when coming into metal box.

  - Using plastic buckets for battery to capture any acid.

  - Heat shrink can be purchased with glue in it.  Sold for wells etc.

- PV’s

  - Heat reduces output.

  - Pole mount helps keep cool and track sun better.

  - Works best to wire to 24 volts quickly from panel.  2-12 panels.  Reduces losses.

- Wind

  - Tower needs to be high!  60, 80 or 100

- Use silicon varistors for lightning protection.

- Inverters Features

  - Specs should be Plus or minus 5 percent rms.  10% peak to peak.

  - Total harmonic distortion <5%  (New Trace 5%.  Excelltech <1 % THD

  - Frequency within .1 Percent

  - Utility Company – 3 days for Pacific Gas and Electric 4-8% THD. <1% THD.  Neighbors, utility boxes etc cause distortion.

  - Modified sine wave (don’t buy) versus true sine wave

  - 60hz operation using FET’s.  Transformer type. Breadbox type

    - suited for running large motors, well pumps, wood shops, freezer.

    - ExcelTec used for 4 years no issues.

  - High Frequency Switching design.  25khz in chopping the signal.

    - Computer electronics, compact florescent lighting work better.

  - Sleep Circuit

    - Inverter only wakes up when load is needed.

  - Overload Protection/Over battery voltage/ Low Battery Voltage Protection. Underwriter Laboratories just insures fire protection..

  - Intertie system is needed for sell back.  Trace is certified.

  - Battery chargers built into the system.

  - Buying too big will waste with small loads.

  - Time of use to charge batteries, meters.

  - Installation

    - Appropriate fuse disconnect between battery and inverter.

  - Trace SW2512

    - 3-5% thd, has an audio hum.

    - real goods 300 units 9 failures a year.

    - X10 doesn’t always work.

    - Has sleep mode.

    - Surge to 3x rated power.

    - Intertie system.  Only one that will stack.

    - Auto starting generator.

    - Battery charger.

    - 98 pounds

  - Prosine by statpower

    - 1.5-3% thd, quiet unless fan.

    - appliances may run different/better with cleaner power.

    - Will not intertie., 2x surge.

    - Good charger.

    - 48 pounds

  - ExcelTech (1kw 700$)

    - Less than 1%.

    - Very good reliability.

    - 32 years mtbf

    - Use for small loads.

    - 2x surge.

    - 11 pounds

  - franklin motor gould pump for his well.

- Questions

  - How is wiring run from stand alone array?