Our boat, Whisper, (from Seattle, had never been winterized before I brought her up north to Minnesota. Needless to say, the first year was a lot of work.

Here are a few principals that work well.

1.      Engine: it’s easier to do this after the boat is pulled, but you can do it in the water while in the lift well if you close the fresh intake seacock. Open your strainer. Open 4-5 gals of antifreeze. We use 100o for the engine, since we don’t want to risk a cracked block. Start the engine and run the water out of the strainer. Start immediately pouring in the antifreeze until you’ve used all bottles. If you’re on the hard, you can see the antifreeze coming out of the exhaust. Turn off the engine. I always top of the strainer and close it. Open the fresh water seacock. Then it’s ready to go in the spring.

2.      Fresh water: I finally put in some valves and lines in to be able to drain and winterize lines. I also put in a bypass for the water heater. Best things I ever did. Here is what I do each year. Empty the water tanks. Bypass the water heater and drain it. Use a 12-volt compressor to blow out any extra water in the lines. Use -60o antifreeze (not pink stuff) and fill all lines. Then use the compressor to blow out any remaining antifreeze in the lines.

I hope you find the winterizing list of use. I’ve attached my latest list. I revise it each time I use it.

If you have more questions, please feel free to email me off-list.

-Bruce (Bruce Pappas)

# Purchase

## Antifreeze: 14 gal -60 & 4 gal -100

## Oil: Rotella-T SAE 15W-40 [8 qts]

## Filter: NAPA 1515 or Wix 51515

## Motor antifreeze: Frostfree LS ethylene glycol

# Exterior

## Pump-out

## Remove sails

## Remove covers

## Remove screens

## Stow/clean dinghy

## Remove dingy engine 
(___self-service or ___yard)

## Remove davits (7/16 wrench & Phillips)

## Remove motor lift

## Remove ring buoy

## Remove EPIRB (take home)

## Remove exterior compass (home)

## Remove BBQ grill

## Remove seats & dry cushions (7/16 socket)

## Lower GPS antenna (5/16 hex)

## Loosen shrouds/stays (looking down – turn clockwise) – ½” wrench & large screwdriver

### Remove screws (9/32 socket & Phillips screwdriver)

### Front shrouds: 4 half turns

### Middle shrouds: 6 half turns

### Back shrouds: 2 half turns

### Rear stay: 6 half turns

## Stow water hose

## Secure lines

# Dinghy engine

## Fog

## Remove gas

## Oil crankcase

## Change sparkplug

# Engine (in water)

## Log engine hours

## Run engine to operating temperature (190oF)

## Drain oil

## Change filter (turn counter-clockwise)

## Add oil (start @ 7 qts & use all 8)

## Check antifreeze to ____ [-34]

## Change fuel filters

### Primary: _______ (yearly)

### Secondary: ________ (10/23/05)

## Wipe down engine bilge

## Check alternator belt

## Check raw water impellor

## Grease pillow block

## Fill water in batteries (distilled)

# Fuel

## Top off tanks, if needed (____ gal each)
Fuel remaining _____ gal each

## Biocide (1 oz/30 gal)

# Webasto heater

## Change fuel filter

## Check antifreeze to -34°

# Air Conditioning (1.5 gal antifreeze)

## Close thru-hull

## Rinse condenser (cockpit)

## Clean & stow filter (with washer)

## Install filter-hose attachment

## Blowout pump (bilge)

## Open AC shut-off

## Blowout condenser (cockpit)

## Replace filter cover

## Antifreeze pump [1 gal]

## Close AC shut-off

## Antifreeze condenser (use funnel) [0.5 gal]

## Remove filter cover

## Spray down rear bilge

# Washdown pump (0.5 gal antifreeze)

## Remove hose

## Open spigot

## Drain line (5/16 socket)

## Antifreeze pump [0.5 gal]

## Clean & stow filter (no washer) – use channel locks

## Spray down front bilge

# Waste/head (3 gal antifreeze)

## Flush head lines

## Flush tank twice

## Flush macerator

## Empty tank

## Antifreeze intake lines & conditioner [5/16 socket for clamp) [0.5 gal]

## Antifreeze tank – from pump-out [1 gal]

## Antifreeze head – pump through lines [1.5 gal]

## Antifreeze macerator (run pump 10 sec)

# Shower pump (before dingy stowage)

## Run shower pump

## Antifreeze line & pump [0.5 gal]

## Clean & stow filter (no washer) – use channel locks

# Frig

## Shut down

## Clean

# Water (5.0 gal antifreeze)

## Turn off water heater breaker (tape)

## Shut off & drain water purifier

## Drain tanks

## Drain water heater (5/16 socket)

## Bypass water heater

## Blowout lines

## Empty galley faucet/spray and head faucet/shower valve

## Drain water purifier & reinstall

## Antifreeze hand/foot pumps [0.75 gal]

## Antifreeze lines [2 gal

## Antifreeze galley faucet/spray

## Antifreeze head faucet/shower valve

## Clean & stow pump filter – use channel locks

## Antifreeze tanks [2 gal each]

# Interior

## Remove binoculars

## Remove first aid kits

## Remove batteries (nav table)

## Remove work freezables: under interior helm, under nav table, under qtr berth

## Remove food freezables (galley)

## Remove head freezables: medicine, suntan lotion, distilled water, etc.

## Stow rugs & blankets

## Remove computer, if needed

## Pack up files as needed

# Propane

## Shut-off tanks

## Disconnect

## MOVE TO WELL

# Out of water (3 gal antifreeze)

## Connect shore power

## Charge batteries

## Open seacocks

## Bilge

### Drain (remove/reinsert garboard plug – plug handle in key drawer)

### Pump dry with hand pump
(pump handle in cockpit locker)

### Antifreeze electric bilge pump [2 gal]

### Antifreeze hand bilge pump (cockpit) [1 gal] - pump dry

## Galley seawater pump

### Pump dry

### Antifreeze from outside [0.25 gal]
(use 7’-0” ¼” ID black tubing)

## Remove garboard plug

## Engine raw water [2¾ gal of -100]

### Run out water first

### Fill strainer & cap [0.25 gal]

## Stow rugs

## Power

### Turn off panel breakers

### Turn off engine compartment breakers

### Turn off Battery Parallel Switch

### Turn off Eng Bat Charge Switch

### Turn off Master

### Disconnect bank of 4 house batteries– check & disconnect (use 9/16 wrench)

### Disconnect bank of 2 starting batteries – check & disconnect (use small adjustable wrench)

### Disconnect shore power and stow cord

## Exterior

### Close blower & heater exhaust vents

### Stow fenders

### Stow dock lines

### Secure halyards

### Touch-up varnish

## Clean cockpit scuppers & cockpit

## Lock cockpit lockers

## Secure whisker pole

## Shrink wrap (yard)

# Spring

## Fill propane tanks

## Wash exterior screens

# Questions

## Cutlass bearing inlet?

## Engine exhaust wet vent

## Provided by Bruce Pappas

## “Whisper” Tashiba 40 Pilothouse

## Duluth, Minnesota

## Addenda:

Raw water strainer is down behind the engine. Mine has a brass cap that screws off and the strainer is underneath. You get there through the opening above the engine and removing the bulkhead panel just aft of the engine.

Regards.

Dave Fierek

The slotted intake you mention is like the reefer intake on my boat.  My raw water strainer is under the lazarette and is the cylindrical brass object in the middle of the photo, directly off the though-hull.  The end unscrews and there’s a strainer inside.  

Allan

## Allan Kaplan

The antifreeze I am talking about is the portable stuff (non-toxic propylene glycol). I agree. I’d never put regular ethylene glycol antifreeze in these systems, like Prestone. It’s all called antifreeze.

-100o: http://www.westmarine.com/webapp/wcs/stores/servlet/producte/10001/-1/10001/32240/377%20710/0/antifreeze/Primary%20Search/mode%20matchallpartial/0/0?N=377%20710&Ne=0&Ntt=antifreeze&Ntk=Primary%20Search&Ntx=mode%20matchallpartial&Nao=0&Ns=0&keyword=antifreeze&isLTokenURL=true&storeNum=5000&subdeptNum=4&classNum=555

-60o: http://www.westmarine.com/webapp/wcs/stores/servlet/producte/10001/-1/10001/71146/377%20710/0/antifreeze/Primary%20Search/mode%20matchallpartial/0/0?N=377%20710&Ne=0&Ntt=antifreeze&Ntk=Primary%20Search&Ntx=mode%20matchallpartial&Nao=0&Ns=0&keyword=antifreeze&isLTokenURL=true&storeNum=5000&subdeptNum=4&classNum=12961

-50o: http://www.westmarine.com/webapp/wcs/stores/servlet/producte/10001/-1/10001/32239/377%20710/0/antifreeze/Primary%20Search/mode%20matchallpartial/0/0?N=377%20710&Ne=0&Ntt=antifreeze&Ntk=Primary%20Search&Ntx=mode%20matchallpartial&Nao=0&Ns=0&keyword=antifreeze&isLTokenURL=true&storeNum=5000&subdeptNum=4&classNum=555

With the cost difference, I don’t use the “pink” stuff (-50) anymore, but the “purple” (-60) stuff instead, except for the engine (-100), which is “blue”.

-Bruce

Just so you know, all the non-toxic (pink and purple stuff) freeze, however, only to a slush not solid. If you read the label it says so. I believe the purple stuff turns to slush at a lower temp then the pink product. This is still considered ok since slush will not damage the water hose, valves or pumps with the exception of the rubber components in the head. For the head I use automotive antifreeze as per the directions from Groco the head manufacturer. I tried cutting corners one winter and deformed the rubber Joker valve and had to replace it. 

There is another alternative you might consider. I have heard of some northeast sailors who use compressed air to blow out all the fresh water lines instead of using pink antifreeze. To do this you need a small compressor and you have to make, (fabricate) an end for the air hose that connects to the cold water lines. Drain the hot water tank and turn on the compressor while opening each faucet on the boat until just air comes out. Don't forget the refrigeration compressor if it is saltwater cooled. You will need to tap into this separately and blow out any left over water. The galley and head foot pumps may need special attention since there should be a check valve in the water system leading to the plumbing for the foot pumps. I believe the best place to tap into the fresh water system with air is on the cold water output of the pump. One other line - the deck washdown, if you have one, must be drained. I have always just disconnected hose at the through hull and drained the pump.

My routine for the engine is to wait until the boat is on the hard. I unscrew the intake stainer cap and put a garden hose inside it. Be sure the intake through hull is closed. Having a small valve on the end of the hose (sold at garden supply store) helps regulate the right amount of water so you don't flood everything. Start water - start your engine at low RPM's and run it until it is up to operating temperature. Shutdown, do an oil change with filter. When complete, restart the engine and check for oil leaks which also refills the new filter with fresh oil. Shut down and top off the engine oil once more. At this point all the salt water has been run out of the raw water side of the engine and you should have fresh water there. Pull the garden hose out and have a few gallons of antifreeze ready. Re-start the engine and immediately start pouring antifreeze into the raw water strainer top. This is easier with a second helper outside the boat watching the exhaust discharge to tell you when it turns pink or whatever color your antifreeze is. Once the color changes at the exhaust shut down the engine until spring. You may want to cover the engine air intake with something to keep moist air from entering and also put tape on all new filters with the date. 

Rich Cassano
S/V Gray Eagle, Tashiba 40
Northport, NY USA

__._,_.___

__===._,_.___